# 4N35, 4N36, 4N37 
Optically Coupled Isolators

## Circuit

## Features

3500, 2500 or 1500 Volt Isolation. 
High DC Current Transfer Ratio (100% min). 
Low Cost Dual-In-Line Package.

## Description

The 4N35, 4N36, 4N37 are optically coupled isolators consisting of a Gallium Arsenide infrared emitting diode and an NPN silicon phototransistor mounted in a standard 6-pin dual-in-line package. Surface Mount Option Available. 
All electrical parameters are 100% tested by manufacturing. Specifications are guaranteed to a cumulative 0.65% AQL.

| ## Absolute Maximum Ratings: (Ta=25°C) |  |
| --- | --- |
| Storage Temperature: Operating Temperature: Lead Soldering: Input-to-Output Isolation Voltage: | -55°C to +150°C -55°C to +100°C 260°C for 10s, 1.6mm from case ±3550Vpeak (4N35) ±2500Vpeak (4N36) ±1500Vpeak (4N37) |
| ### Input Diode |  |
| Forward DC Current: Reverse DC Voltage: Peak Forward Current: Power Dissipation: Derate Linearly: | 60mA 6V 3A (t p=10µs) 100mW 1.33mW/°C above 25°C |
| ### Output Transistor |  |
| Collector-Emitter Voltage: Emitter-Collector Voltage: Collector-Base Voltage: Power Dissipation: Derate Linearly: | 30V 7V 70V 300mW 2.00mW/°C above 25°C |
| ### Package |  |
| Total Power Dissipation: Derate Linearly: | 400mW 3.3mW/°C above 25°C |

## Electro-optical Characteristics: (Ta=25°C)

| INPUT | PARAMETER | CONDITIONS | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| VF | Forward Current | IF=10mA | 0.8 |  | 1.5 | V |
|  |  | IF=10mA, TA=-55°C | 0.9 |  | 1.7 | V |
|  |  | IF=10mA, TA=100°C | 0.7 |  | 1.4 | V |
| VR | Reverse Breakdown Voltage | IR=10µA | 6 |  |  | V |
| IR | Reverse Current | VR=6V |  |  | 10 | µA |
| OUTPUT | PARAMETER | CONDITIONS | MIN | TYP | MAX | UNIT |
| BVCEO | Collector-Emitter Voltage | IC=1mA | 30 |  |  | V |
| BVECO | Emitter-Collector Voltage | IE=100µA | 7 |  |  | V |
| BVCBO | Collector-Base Voltage | IC=100µA | 70 |  |  | V |
| ICEO | Collector-Emitter Dark Current | VCE=10V, IB=0, TA=25°C |  | 5 | 50 | nA |
| ICEO | Collector-Emitter Dark Current | VCE=30V, IB=0, TA=100°C |  |  | 500 | µA |
| ICBO | Collector-Base Dark Current | VCB=10V, IE=0 |  |  | 20 | nA |
| CCE | Collector-Emitter Capacitance | VCE=10V, f=1MHz | 2 | 10 |  | pF |
| HFE |  | VCE=5.0V, IC=100µA | 100 | 150 |  |  |
| COUPLED | PARAMETER | CONDITIONS | MIN | TYP | MAX | UNIT |
| IC/IF | DC Current Transfer Ratio | IF=10mA, VCE=10V, TA=25°C | 100 |  |  | % |
|  |  | IF=10mA, VCE=10V, TA=100°C | 40 |  |  | % |
|  |  | IF=10mA, VCE=10V, TA=-55°C | 40 |  |  | % |
| VCE(SAT) |  | IF=10mA, IC=0.5mA, IB=0 | 10 |  |  | % |
| RIO | Input-to-Output Isolation Resistance | VIO=500V, (note 1) | 1E11 |  |  | ohm |
| CIO | Capacitance Input to Output | f=1MHz (note 1) |  | 1 |  | pf |
| TR | Output Rise Time | VCC=10V, IC=2mA |  | 3 |  | µs |
| TF | Output Fall Time | RL=100ohm |  | 3 |  | µs |
|  | Input-Output Isolation Voltage |  |  |  |  |  |
|  | 4N35 | (Note 1) | 3550 |  |  | V |
|  | 4N36 |  | 2500 |  |  | V |
|  | 4N37 |  | 1500 |  |  | V |