# BT-12S

| # BT-12S |  |
| --- | --- |
| Size : L x W x H ( mm ) | 20 * 9.8 * 11 |
| Featured | DIL Pitch terminal High Reliability-Bifurcated Contact |
| Contact arrangement | 2 Form C |
| Contact Material | Ag-Pd (Au Clad) |
| Contact Rating Rated Load ( Resistive ) | 2A30V DC 1A125V AC |
| Motor Load | -- |
| Max. Switching Current | 2A |
| Max. Switching Voltage | 250V AC ; 220V DC |
| Max. Switching Capacity | -- |
| Min. Switching Load | -- |
| Sealed type available | * |
| Life (Min, operation) Electrical (at rated load) Mechanical (no load) | 300,000 operations (2A30V DC) 1,000,000 operations (1A30V DC) 100,000,000 operations |
| Breakdown voltage Between open contact Between contact and coil | 1000V AC 1500V AC |
| Surge Strength | 1500V |
| Coil voltage DC AC | 3,5,6,9,12,24,48V DC -- |
| Nominal Consumption | -- |
| Terminal Layout (bottom view) |  |
| Standards | UL, CSA |